Appendix B: MVCL Rule Set

The rule set below resolves the type, cardinality, and navigation inconsistencies identified during the audit of the previous draft. The intended distinction between syntactic, structural, and semantic inconsistency detection is preserved.

B.1 Syntactic Rules (syntactic_rules.mvcl)

// SR-01: Required attributes

rule SR_01_AttributeExistence : syntactic

context ElectricalComponent ec

check ec.id <> null and ec.name <> null

// SR-02: Non-negative requirement capacity

rule SR_02_TypeCompatibility : syntactic

context Requirement r

check r.minRequiredCapacity >= 0.0

// SR-03: Enumeration validity

rule SR_03_EnumValidity : syntactic

context ProductionUnit pu

check pu.status <> null

// SR-04: Reference validity

rule SR_04_ReferenceValidity : syntactic

context MechanicalPart mp

check mp.linkedController <> null implies mp.linkedController.id <> null

// SR-05: Battery attribute-domain consistency

rule SR_05_DataTypeConsistency : syntactic

context Battery b

check b.capacity >= 0.0 and b.chemistry <> null

// SR-06: Software component completeness

rule SR_06_SoftwareCompleteness : syntactic

context SoftwareComponent sc

check sc.vendor <> null and sc.version matches "[0-9]+\.[0-9]+\.[0-9]+"

// SR-07: Production unit completion

rule SR_07_ProductionCompleteness : syntactic

context ProductionUnit pu

check pu.startDate <> null and pu.capacity > 0

// SR-08: Controller firmware validity

rule SR_08_FirmwareValidity : syntactic

context Controller c

check c.firmwareVersion matches "[0-9]+\.[0-9]+"

B.2 Structural Rules (structural_rules.mvcl)

// ST-01: Required sensor connections

rule ST_01_RequiredConnection : structural

context Controller c

check c.linkedSensor->size() > 0

// ST-02: Production-line completeness

rule ST_02_LineCompleteness : structural

context ProductionLine pl

check pl.units->size() = pl.parts->size()

// ST-03: Controller mapping uniqueness

rule ST_03_ReferenceUniqueness : structural

context MechanicalPart mp

check MechanicalPart.allInstances()->isUnique(p | p.linkedController)

// ST-04: Bidirectional part-line consistency

rule ST_04_BidirectionalConsistency : structural

context ProductionLine pl

check pl.parts->forAll(p | p.usedIn = pl)

// ST-05: Hardware-controller mapping

rule ST_05_HardwareSoftwareMapping : structural

context ElectricalComponent ec

check ec.connectedTo->forAll(c |

c.type = ComponentType::CONTROLLER implies c.id <> null)

// ST-06: Controller-sensor linkage

rule ST_06_ControllerSensorLink : structural

context Controller c

check c.linkedSensor->forAll(s | s.connectedTo = c)

// ST-07: Requirement assignment

rule ST_07_RequirementAssignment : structural

context Requirement r

check r.assignedTo <> null

// ST-08: Dependency graph acyclicity

rule ST_08_DependencyAcyclicity : structural

context Functionality f

check f.dependsOn->closure(d | d.dependsOn)->excludes(f)

// ST-09: Production unit-line relationship

rule ST_09_ProductionUnitLine : structural

context ProductionUnit pu

check pu.line <> null and pu.line.units->includes(pu)

// ST-10: Part-line relationship

rule ST_10_PartLineRelationship : structural

context MechanicalPart mp

check mp.usedIn <> null and mp.usedIn.parts->includes(mp)

B.3 Semantic Rules (semantic_rules.mvcl)

// SE-01: Battery capacity adequacy

rule SE_01_BatteryCapacity : semantic

context Battery b, Requirement r

check b.capacity >= r.minRequiredCapacity

// SE-02: Power-supply adequacy with safety factor

rule SE_02_PowerSupplyAdequacy : semantic

context PowerSupply ps, SystemLoad sl

check ps.maxPower >= sl.peakLoad * 1.2

// SE-03: Production readiness

rule SE_03_ProductionReadiness : semantic

context ProductionUnit pu

check (pu.status = ProductionStatus::PRODUCTION)

implies (pu.validated = true and pu.capacity > 0)

// SE-04: Effective software-hardware capacity alignment

rule SE_04_CapacityAlignment : semantic

context Requirement r, Battery b

check r.minRequiredCapacity <= b.capacity * b.health / 100

// SE-05: Validation timing constraint

rule SE_05_ValidationTiming : semantic

context Controller c

check daysBetween(c.deploymentDate, c.lastValidationDate) <= 30

// SE-05 uses an implementation-defined daysBetween helper that returns elapsed calendar days; the helper is provided by the MVCL-to-EVL mapping layer to avoid relying on implicit date-minus-integer coercion.
// SE-06: Production-line efficiency

rule SE_06_LineEfficiency : semantic

context ProductionLine pl

check pl.efficiency >= 75.0

// SE-07: Performance requirement versus sensor accuracy

rule SE_07_SensorAccuracy : semantic

context Sensor s, Requirement r

check (r.type = RequirementType::PERFORMANCE and

r.assignedTo = s.connectedTo)

implies s.accuracy >= 95.0

// SE-08: Battery lifecycle constraint

rule SE_08_BatteryLifecycle : semantic

context Battery b

check b.cycles < 1000

// SE-09: MUST_HAVE requirement has a versioned assigned component

rule SE_09_RequirementPriority : semantic

context Requirement r

check (r.priority = RequirementPriority::MUST_HAVE)

implies (r.assignedTo.version <> null)

// SE-10: Production-unit location validity

rule SE_10_LocationValidity : semantic

context ProductionUnit pu

check pu.location = pu.line.location

// SE-11: Controller sensor coverage

rule SE_11_SensorCoverage : semantic

context Controller c, SystemLoad sl

check c.linkedSensor->size() >=

(sl.priority = LoadPriority::CRITICAL ? 2 : 1)

// SE-12: Power-supply redundancy

rule SE_12_PowerRedundancy : semantic

context SystemLoad sl

check sl.connectedTo->size() >=

(sl.priority = LoadPriority::CRITICAL ? 2 : 1)

B.4 Tolerance Context Definitions (contexts.json)

{

"contexts": [

{

"id": "prototype-phase",

"description": "Development prototype phase with relaxed constraints",

"parameters": {

"phase": "Prototype",

"criticality": "Low",

"deadline": "2026-12-31",

"team_size": 5,

"budget": 1000000

}

},

{

"id": "production-phase",

"description": "Production phase with strict constraints",

"parameters": {

"phase": "Production",

"criticality": "High",

"deadline": "2026-09-01",

"team_size": 20,

"budget": 10000000

}

},

{

"id": "maintenance-phase",

"description": "Maintenance phase with balanced constraints",

"parameters": {

"phase": "Maintenance",

"criticality": "Medium",

"deadline": "2026-12-01",

"team_size": 8,

"budget": 3000000

}

}

]

}

The numerical tolerance thresholds for SE-01–SE-12 remain in the machine-readable contexts.json artifact used by the tolerance module. They are intentionally separated from the detection rules, preserving the detection–tolerance separation adopted by the framework.